## NENA-2 Data Record Specifications

The following section contains the Data Record, Header Record, and Trailer Record specifications based on the National Emergency Number Association (NENA) recommended format for data exchange - Version 2.

| Field Name | Positions | Bytes | Type | Req’d | Description |
| --- | --- | --- | --- | --- | --- |
| Function Code | 1 | 1 | A | R | The type of activity the record is being submitted for. Valid entries are: C = Change, D= Delete, I = Insert U= Unlock, M=Migrate |
| NPA | 2 - 4 | 3 | N | R | The three digit area code of the Calling/Working Number. |
| Calling Number | 5 - 11 | 7 | N | R | The seven digit telephone number of the Calling/Working Number. |
| House Number | 12 - 21 | 10 | AN | R/SF | The house number. This field should be spaced filled if no house number is available. |
| House Suffix Number | 22 - 25 | 4 | AN | R/SF | ***NEED TO DISCUSS*** The house number extension. (e.g. 1/2) This field should be space filled if no suffix applies. |
| Prefix Directional | 26 - 27 | 2 | A | R/SF | The leading street direction prefix. Valid entries are: N, S, E, W, NE, NW, SE, SW. This field should be space filled if no prefix applies. |
| Street Name | 28 - 87 | 60 | AN | R | ***NEED TO DISCUSS*** The service address of the Calling Number. |
| Street Suffix | 88 - 91 | 4 | A | R/SF | ***NEED TO DISCUSS*** The street abbreviation, as defined by the U. S. Postal Service Publication 28. (e.g. AVE) This field should be space filled if no suffix applies. |
| Post Directional | 92 - 93 | 2 | A | R/SF | The trailing street direction suffix. Valid entries are: N, S, E, W, NE, NW, SE, SW. This field should be space filled if no suffix applies. |
| Community Name | 94 - 125 | 32 | A | R | The service community name of the street / house number as assigned by the Municipal Street Address Guide. |
| State | 126 - 127 | 2 | A | R | The alphabetic state abbreviation as defined by the U. S. Postal Service. (e.g. PA) |
| Location | 128 - 187 | 60 | AN | R/SF | The additional address information describing the exact location of the Calling Number. (e.g. SUITE 2002, APT 706A) This field should be space filled if no location applies. See Notes for additional information on the use this field. |
| Customer Name | 188 - 219 | 32 | AN | R | The subscriber name associated with the Calling Number. |

NENA-2 Data Record Specifications (continued)

| Field Name | Positions | Bytes | Type | Req’d | Description |
| --- | --- | --- | --- | --- | --- |
| Class of Service | 220 | 1 | AN | R | Valid entries are: 1 = Residence 2 = Business 3 = Residence Private Branch Exchange (PBX) 4 = Business PBX 5 = CENTREX 6 = Coin 1 way out 7 = Coin 2 way 8 = Mobile 9 = Residence Off Premise Extension (OPX) 0 = Business OPX See Notes for additional information on the use this field. |
| Type of Service | 221 | 1 | N | R | Valid entries are: 0 = Not Foreign Exchange (FX) or Non Published (Non Pub) 1 = FX in 911 serving area 2 = FX outside 911 serving area 3 = Non Pub 4 = Non Pub FX in 911 serving area 5 = Non Pub FX outside 911 serving area |
| Exchange | 222 - 225 | 4 | AN | R/SF | ***NEED TO DISCUSS*** Field not used, space fill. |
| ESN | 226 - 230 | 5 | AN | N/SF | Field not used, space fill. |
| Main NPA | 231 - 233 | 3 | N | R +/or SF | The three digit area code of the Main Number associated with the Calling Number. |
| Main Number | 234 - 240 | 7 | N | R +/or SF | The seven digit telephone number of the Main Number associated with the Calling Number |
| Order Number | 241 - 250 | 10 | AN | R/SF | The service order number for the activity establishing this record. |
| Extract Date | 251 - 256 | 6 | N | R +/or SF | The date on which the record was created in month, day, year, format (e.g. MMDDYY). |
| County ID | 257 - 260 | 4 | AN | N/SF | Field not used, space fill. |
| Company ID | 261 - 265 | 5 | AN | R | The Telephone Company Identification code. This field is to be populated using the NENA Company Code. |

NENA-2 Data Record Specifications (continued)

| Field Name | Positions | Bytes | Type | Req’d | Description |
| --- | --- | --- | --- | --- | --- |
| Source ID | 266 | 1 | AN | R +/or SF | A code that indicates whether the data is part of an initial data load or re-load or part of a daily update process. Valid values are: Space = Daily Update, C = Load |
| Zip Code | 267 - 271 | 5 | AN | N/SF | The five digit Postal Zip Code. |
| Zip + 4 | 272 - 275 | 4 | AN | N/SF | Field not used, space fill. |
| General Use | 276 - 286 | 11 | AN | N/SF | Field not used, space fill. |
| Customer Code | 287 - 289 | 3 | AN | N/SF | Field not used, space fill. |
| Comments | 290 319 | 30 | AN | N/SF | Field not used, space fill. |
| X Coordinate | 320 - 328 | 9 | N | N/SF | Field not used, space fill. |
| Y Coordinate | 329 - 337 | 9 | N | N/SF | Field not used, space fill. |
| Z Coordinate | 338 - 342 | 5 | N | N/SF | Field not used, space fill. |
| Cell ID | 343 - 348 | 6 | AN | N/SF | Field not used, space fill. |
| Sector ID | 349 | 1 | AN | N/SF | Field not used, space fill. |
| TAR code | 350 - 355 | 6 | AN | R | Taxing Area Rate code. See Notes for additional information on the use this field. |
| Reserved | 356-376 | 21 | AN | N/SF | This field is reserved for the Data Base Management Systems provider’s use. |
| Alternate Number | 377-386 | 10 | N | R/SF | Customer Number being remote call forwarded in Interim Number Portability service. |
| Expanded Extract Date | 387-394 | 8 | N | R | Date on which the record was created in the format YYYYMMDD |
| NENA reserved | 395-480 | 86 | AN | N | This field is reserved for NENA Data Technical Committee Assignment. |
| Reserved | 481-511 | 31 | AN | N/SF | This field is reserved for the Data Base Management Systems Provider’s use. |
| End of Record | 512 | 1 | AN | R | End of record indicator. Always contains an asterisk (*). |

NENA-2 Data Record Specifications (continued)

Notes:

1. Req’d (required) column

R	=  Required field.  Must be populated for every subscriber’s record spaces not permitted.

R/SF	=  Required field.  Must be populated  if present in subscriber’s record otherwise space fill.

N/SF	=  Not required field.  Space fill.

2.  All fields are left justified with trailing spaces.

3.  The actual lengths of some fields on the 911 systems (e.g. Street, Community, Location, et al) are shorter than the NENA-2 defined length and vary by system which may result in truncation of rightmost data.  The most significant information should always be left justified in these and all fields.

4.  All AN (Alphanumeric) fields may contain Upper case letters, Numbers and punctuation characters.  Lower case and control characters are not permitted.